TO:    ALL WOOD COUNTY ARES MEMBERS  (OFFICIAL)

ALL ARES AND NON-ARES AMATEURS ANYWHERE  (INFORMATION)

FROM:  KEN HARRIS WA8LLM ARRL DISTRICT 3

EMERGENCY COORDINATOR WOOD COUNTY EMERGENCY COMMUNICATIONS INC.

WOOD COUNTY ARES BULLETIN NR 21.23      DATE: June 6, 2021

SUBJECT: IT MAY HAVE TO COME DOWN

When you put up a tower do you plan for the day that it may have

to be taken back down?

I've installed many towers since becoming a Amateur Radio

operator, and each time I put one up, I try to plan for the day

that it may have to come back down.  There are several things

that you can do that will make taking it back down a whole lot

easier.

When installing the tower rustproof the bolts with a good quality

rust inheritor.  Attach the guy wires above the top step of the

tower section, not the middle or bottom step of an upper section.

Run transmission lines on the tower so they are out of the way of

climbing the tower, and easy to secure to the tower.  Don't run

the coax or transmission line inside of the tower.  Attach each

antenna to the tower using it's own mounting bracket so you can

take down just one antenna at a time, and not have to worry about

any other one.

Make taking a tower down a whole lot easier, and safer than

putting it up.  Plan ahead, and think what you are doing.

Ken Harris WA8LLM

WCEC

Wood County WV