================================================================================

DAREN of WV News November 13, 2022                   (F84)

(304) 679-3470 wa8llm@yahoo.com

Wood County Emergency Communications, 319 Communications Dr, Waverly, WV 26184

www.wc8ec.com or www.wc8ec.org

DAREN News, are articles and information about the Digital Amateur Radio

"Emergency" Network of WV, which operates on 145.69 MHz.  Anyone having any

information about DAREN, or Amateur Radio in general, and would like to share

it with others, may send it to WA8LLM @ PKBGWV on 145.69 MHz, or to my E-Mail:

wa8llm@yahoo.com.  A State Wide DAREN NET is operated every Saturday from 7:00

pm to 10:00 pm, and lately, before and after those hours.  The DAREN Net

Announcement, showing the MAIN NCS, ALTERNATE NCS, NODEs accessible to both,

and the previous week's check-ins, are posted by Thursday on most of the Main

county DAREN PBBS'.  The DAREN News can also be found on the www.wc8ec.com and

www.wc8ec.org websites.

Well, we didn't do as much as we planned on Sammy Mercer, N8SVX's, tower

yesterday.  The weather didn't start out too bad, but by the time we got 40 feet

of tower up, the rain was getting too cold to do anything but get warm and dry.

Since there's not much news, I think I'll explain few packet radio commands over

the next few weeks.  The commands may help you set up your local and remote

DAREN stations.  Some of the commands can be used over the air.  All of the

commands can be used with a computer attached to the TNC (Terminal Node

Controller).

The use of the <ctrl> "V" which is a pass code can be used to allow you to press

the <enter> key without the cursor jumping to the next line, and entering the

command.  The <ctrl> V can not be used over the air to a remote station.

If you want to make your PBBS (Personal Bulletin Board System) have room for

several messages, all you need to do is type the amount of memory you want

allotted.  Example type "PBBS" <enter> and it will give you the amount of memory

allotted to the PBBS.  If it says "7", that means that it has 7000 bits of

memory for messages.  If it says "10", then it has 10,000 bits of memory for

messages.  If it says "100" that means it has 100,000 bits for messages.  To

change the allotted memory, just type in "PBBS" followed by the new amount.  If

you want to re-number the messages in the PBBS, just type "PBBS" followed by the

number of bits that is already allotted.  For example if it says "100" just type

"PBBS 100".  When you press the <enter> key it will re-number all of the

messages in the PBBS starting with 1.

There's a good command that will allow you to stay at the NODE you are already

at when you want to re-connect to another NODE, after you disconnect from the

one you are connected to.  When you are at a NODE connecting to another NODE

just add the letter "S" after the NODE name, "S" stands for STAY.  For example,

if you are at WOODWV, and you are connecting to RITCWV, but you want to stay a

WOODWV after you disconnect from RITCWV, and you want to re-connect to WIRTWV,

add the letter "S" after RITCWV.  For example "C RITCWV S" <enter>.  When you

disconnect from RITCWV, you will stay at WOODWV, and you're ready to re-connect

to WIRTWV.

I'll have some more commands next week.  Hopefully I don't get you confused.

There were 37 check-ins to the State Wide DAREN Net this week.  Delmas Hartley,

N8JYS, needs only one more check-in to make 450.

================================================================================