================================================================================

DAREN of WV News November 20, 2022                   (F85)

(304) 679-3470 wa8llm@yahoo.com

Wood County Emergency Communications, 319 Communications Dr, Waverly, WV 26184

www.wc8ec.com or www.wc8ec.org

DAREN News, are articles and information about the Digital Amateur Radio

"Emergency" Network of WV, which operates on 145.69 MHz.  Anyone having any

information about DAREN, or Amateur Radio in general, and would like to share

it with others, may send it to WA8LLM @ PKBGWV on 145.69 MHz, or to my E-Mail:

wa8llm@yahoo.com.  A State Wide DAREN NET is operated every Saturday from 7:00

pm to 10:00 pm, and lately, before and after those hours.  The DAREN Net

Announcement, showing the MAIN NCS, ALTERNATE NCS, NODEs accessible to both,

and the previous week's check-ins, are posted by Thursday on most of the Main

county DAREN PBBS'.  The DAREN News can also be found on the www.wc8ec.com and

www.wc8ec.org websites.

I hope everyone has a Happy and Safe Thanksgiving.  If you are a hunter I hope

you are lucking enough to get the size deer you want and be safe in the woods.

I'll give some information about a couple commands that may be of some use to

set up a TNC (Terminal Node Controller).  Access to any remote stations that you

own may save you a trip to a hilltop or mountain top station.  The first thing

you must do is give your TNC a remote access alias name or callsign by using the

command of MYRemote or just MYR.  The next thing you must do is place a string

of letters, or a phrase in the RText or RT command.  The RText can also be used

for accessing the SYSOP commands in your PBBS (Private Bulletin Board System).

When you connect to the MYRemote call or alias, the TNC will send you back three

sets of random numbers, each line with 6 numbers.  You must pick one of the

lines and decode the password string by placing a string of letters that

represent each of the numbers.  Remember that case is significant and spaces are

considered valid characters.  An example RText might be;

A B C D E F G H I J  K  L  M  N  O  P  Q  R  S  T  U  V  W  X  Y  Z

1 2 3 4 5 6 7 8 8 10 11 12 13 14 15 16 17 18 18 20 21 22 23 24 25 26

When the letters you send match the password string are correct, the TNC will

give a command prompt back, and you will have access to your TNC.  If you don't

get the password right, the remote station TNC will give you two more tries,

before locking you out of the remote station for 15 minutes.  The best way to

disconnect from the remote station is to do a RESET command.  The RESET command

will disconnect you from the remote station, and it may clear any problems that

it has built up since the last time it was reset.

A command that may help you transmit Packet data is the TXdelay or just TX.  The

TXdelay allows your transceiver to switch from receive to transmit, and full

power, before any data is sent over the air from the TNC.  A rule thumb I use is

to set the TXdelay to 35, which represents 350 milliseconds.  If the TNC starts

sending data before the transmitter on and up to full power, some of the TNC

commands or traffic may not be sent over the air.

There were 37 check-ins to the State Wide DAREN Net again this week.  Delmas

Hartley, N8JYS, who has been participating in the net since January 14, 1995,

has completed 450 check-ins and will be receiving his certificate.  Jeffrey

Hensley, WD4VA, needs only one more check-in to make 500.

================================================================================